### Choice #1:  identify major fields and provide values below (this is most similar to how MS presents this kind of information in [MS-SOH])

| 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | R | TLV Type |  |  |  |  |  |  |  |  |  |  |  |  |  | Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IANA SMI Vendor Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component ID |  |  |  |  |  |  |  |

M: The M bit must be set to 0.

R: The R bit is reserved, and must be set to 0

TLV Type: The TLV Type for this packet type must be set to 2.

Length: The Length field for this packet type must be set to 4.

IANA SMI Vendor Code: The Vendor Code must be set to the PWG SMI, which is 2699 (0xA8B).

Component ID: This 8 bit field identifies the component type, qualified by the Vendor Code.  The following values are defined for hard copy devices.  Values are additive (for example, printer + scanner + copier = 7).

1 = Printer Device
2 = Scanner Device
4 = Copier Device
8 = Fax Device

Values 16, 32, 64, and 128 are reserved and must be set to 0. Therefore, valid values for Component ID range from 0x01 to 0x0F.

### Choice #2: identify major and minor fields and provide values below (might be a problem to make unique identifiers for long bit fields)

| 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | R | TLV Type |  |  |  |  |  |  |  |  |  |  |  |  |  | Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IANA SMI Vendor Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component ID |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R | R | R | R | P | S | C | F |

M: The M bit must be set to 0.

R: The R bits are reserved, and must be set to 0

TLV Type: The TLV Type for this packet type must be set to 2.

Length: The Length field for this packet type must be set to 4.

IANA SMI Vendor Code: The Vendor Code must be set to the PWG SMI, which is 2699 (0xA8B).

Component ID: This 8 bit field identifies the component type, qualified by the Vendor Code.  The following values are defined for hard copy devices.  Values are additive (for example, printer + scanner + copier = 7).

P = 1 for a Printer device, otherwise P = 0
S = 1 for a Scanner device, otherwise S = 0
C = 1 for a Copier device, otherwise C = 0
F = 1 for a Fax device, otherwise F = 0

The remaining bits in the Component ID field are Reserved.

### Choice #3: identify fields and provide embedded values where possible, otherwise provide values below

| 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | TLV Type = 2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Length = 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IANA SMI Vendor Code = 0xA8B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component ID |  |  |  |  |  |  |  |

Component ID: This 8 bit field identifies the component type, qualified by the Vendor Code.  The following values are defined for hard copy devices.  Values are additive (for example, printer + scanner + copier = 7).

1 = Printer Device
2 = Scanner Device
4 = Copier Device
8 = Fax Device

16, 32, 64, and 128 are reserved and must be set to 0. Therefore, valid values for Component ID range from 0x01 to 0x0F.

### Choice #4~n: your suggestions?