# IPP Everywhere 1.0

Status: Interim

Abstract: This standard defines an extension of IPP to support network printing without vendor-specific driver software, including the transport, various discovery protocols, and standard document formats.

This document is a PWG Working Draft. For a definition of a "PWG Working Draft", see: ftp://ftp.pwg.org/pub/pwg/general/pwg-process30.pdf

This document is available electronically at:

ftp://ftp.pwg.org/pub/pwg/ipp/wd/wd-ippeve10-20110927.docx

ftp://ftp.pwg.org/pub/pwg/ipp/wd/wd-ippeve10-20110927.pdf

Copyright © 2011 The Printer Working Group. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  IPP Everywhere 1.0

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at: ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary. The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit:

http://www.ieee-isto.org

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:

http://www.pwg.org

Contact information:

The Printer Working Group

c/o The IEEE Industry Standards and Technology Organization

445 Hoes Lane

Piscataway, NJ 08854

USA

About the Internet Printing Protocol Work Group

The Internet Printing Protocol (IPP) working group has developed a modern, full-featured network printing protocol, which is now the industry standard. IPP allows a print client to query a printer for its supported capabilities, features, and parameters to allow the selection of an appropriate printer for each print job. IPP also provides job information prior to, during, and at the end of job processing.

For additional information regarding IPP visit:

http://www.pwg.org/ipp/

Implementers of this specification are encouraged to join the IPP mailing list in order to participate in any discussions of the specification. Suggested additions, changes, or clarification to this specification, should be sent to the IPP mailing list for consideration.

# Table of Contents

1. Introduction	8

2. Terminology	8

2.1 Conformance Terminology	8

2.2 Imaging Terminology	8

2.3 Other Terminology	9

3. Requirements	10

3.1 Rationale for IPP Everywhere	10

3.2 Use Cases	11

3.2.1 Select Printer	11

3.2.2 Print	13

3.2.3 Exceptions	16

3.3 Out of Scope	18

3.4 Design Requirements	18

4. Discovery Protocols	19

4.1 Printer Description Attributes Used in Discovery	19

4.2 DNS Service Discovery	20

4.2.1 Service Instance Name (SRV)	20

4.2.2 Geo-Location (LOC)	20

4.2.3 Service Information (TXT)	20

4.3 LDAP and SLP Discovery	24

4.4 SSDP Discovery	24

4.4.1 Device Definitions	26

4.4.2 Theory of Operation	26

4.4.3 XML Device Description	27

5. Protocol Binding	29

5.1 IPP Operations	29

5.2 IPP Printer Description Attributes	29

5.3 IPP Operation Attributes	32

5.4 IPP Job Template Attributes	32

5.5 IPP Job Description Attributes	34

6. Document Formats	36

7. Additional Values for Existing Attributes	36

7.1 ipp-extensions-supported (1setOf type2 keyword)	36

8. Conformance Requirements	37

8.1 Conformance Requirements for this Specification	37

8.2 Conditional Conformance Requirements for Printer Objects	37

9. Internationalization Considerations	37

10. Security Considerations	38

11. IANA Considerations	38

11.1 Attribute Value Registrations	38

12. References	38

12.1 Normative References	38

12.2 Informative References	41

13. Authors' Addresses	41

14. Change History	43

14.1 September 27, 2011	43

14.2 August 3, 2011	43

14.3 March 16, 2011	43

# List of Figures

Figure 1 - UPnP Printer:2 Functional Diagram	25

# List of Tables

Table 2 - DNS TXT Record Keys	20

Table 3 - Device Requirements for urn:schemas-upnp-org:printer:2	26

Table 5 - Printer Description Attributes	29

Table 7 - Job Template Attributes	32

Table 8 - Job Description Attributes	34

Introduction

New mobile devices (e.g., cellphones, PDAs, netbooks, etc.) do not follow the traditional use models for printing services. For mobile devices, discovery of available printers and their capabilities is both more difficult than for traditional desktop systems and more important because of dynamically changing network attachment points.

Printer vendors and software vendors have defined and deployed many different document formats (page description languages) and also dialects of those document formats, increasing the traditional desktop system need for model-specific printer drivers. While there are millions of model-specific printer drivers now available for traditional desktop systems, this printer driver model is clearly not practical for mobile devices.

The goal of IPP Everywhere is to allow Clients, particularly mobile Internet devices, to easily support printing using IPP but without the use of vendor-specific drivers through the adoption of standard document formats, discovery protocols, and schemas.

Terminology

Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, and OPTIONAL, have special meaning relating to conformance as defined in RFC 2119 [RFC2119].

Imaging Terminology

Normative definitions and semantics of printing terms are imported from IETF Printer MIB v2 [RFC3805], IETF Finisher MIB [RFC3806], and IETF IPP/1.1 [RFC2911].

This document also defines the following protocol roles in order to specify unambiguous conformance requirements:

Device; IPP definition; also see Logical Device and Physical Device

Client; Initiator of outgoing IPP session requests and sender of outgoing IPP operation requests (HTTP/1.0 Client [RFC1957] / HTTP/1.1 Client [RFC2616]).

Printer; Listener for incoming IPP session requests and receiver of incoming IPP operation requests (HTTP/1.0 Server [RFC1957] / HTTP/1.1 Server [RFC2616]).

Imaging Device; A printer or other device that acts as an IPP Printer.

Job; A queued unit of work such as a document for printing.

Other Terminology

Coloring; Filtering of a response or results based on one or more attribute values.

Direct Imaging; Printing, facsimile, and scanning performed by direct communication from the Client to an Imaging Device or local print server.

Directory Service; A Service providing query and enumeration of information using names or other identifiers.

Discovery; The process of querying or browsing local network segments for Imaging Devices such as Printers or their services.

Discovery Protocol; A (typically) connectionless protocol for enumeration of local Devices.

Enumeration; The process of listing Imaging Devices that are registered with a Directory or other Service.

Indirect Imaging; Printing, facsimile, and scanning performed by communication from the Client and/or Imaging Device to an intermediary service in a different administrative domain, for example when the Client communicates with a third-party print service or when an Imaging Device communicated with a Cloud service.

Operator; A person or automata responsible for maintaining and/or controlling a Device.

Service; Software providing access to physical, logical, or virtual resources and (typically) processing of queued Jobs.

User; A person or automata using a Device.

Visible Device; A Device that can be directly accessed by a Client.

Visible/Visibility; Refers to the ability of one device to communicate directly with another, for example a Client is able to connect to a Device, query for supported attributes, submit print jobs, and so forth.

Requirements

Rationale for IPP Everywhere

Given the following existing specifications and the need for a standard method of Direct Imaging without traditional vendor-specific driver software, the IPP Everywhere Specification should:

- Use existing protocols and schema to support discovery, identification, and auto-configuration of Imaging Devices,

- Use the existing IPP specifications to support job submission to and monitoring of Imaging Devices,

- Encourage support for printing through standard document formats, and

- Discourage the further proliferation of vendor-specific page description languages, formats, discovery protocols, interfaces, and transports

The Internet Printing Protocol Version 2.0 Second Edition [PWG5100.12] defines:

A collection of existing IPP specifications that form the basis for IPP/2.0

Standard job template attributes

Specific interoperability requirements, such as HTTP/1.1 support with chunking and IPP collection attribute support

New version number and operation requirements for different classes of devices

The IPP: Job and Printer Extensions - Set 3 [PWG5100.JPS3] define new attributes and operations required for mobile and "driverless" printing.

The PWG Raster Format [PWG5102.RAS] defines a minimal file format for transmission of multi-page color and grayscale bitmap images

The Document management -- Portable document format -- Part 1: PDF 1.7 [ISO32000] defines:

- A rich file format for transmission of multi-page color and grayscale vector and bitmap images

Standard page attributes to support page size, orientation, and duplex functionality

The JPEG File Interchange Format Version 1.02 [JFIF] defines a compact file format for transmission of photographic images

The Open XML Paper Specification [ECMA388] defines a paginated document format based on OPC, XML, and standard image and font formats with device-independent color.

The Bonjour Printing Specification version 1.0.2 [BONJOUR] defines:

- Discovery of IPP printers using DNS SRV lookups [RFC2782]

- Multicast DNS for use on link-local networks [MDNS]

Automatic address assignment for both IPv4 [RFC3927] and IPv6

DNS TXT record keys to support auto-configuration, capabilities, identification, and protocol selection

The Lightweight Directory Access Protocol (LDAP): Schema for Printer Services [RFC3712] defines a schema for printer registrations and discovery via LDAP and SLP services.

Use Cases

Select Printer

Printer selection is part of most Service use cases - the User selects a Printer, implicitly or explicitly, and the remainder of the use case applies to the selected device. A Printer can be a Logical Printer such as a Scan Service or a Physical Printer such as a Printer. Selection use cases can often be combined, for example Selection Using a Directory Service (section 3.2.1.4) with Selection Using Properties (section 3.2.1.9).

In order to simplify the selection use cases, common exceptions are listed as separate use cases in section 3.2.3.

Precondition: For all of the following use cases, the Printer must be Visible to the Client to be selected, either directly or through an intermediate Service.

Select the Last Used Printer

The Client User Interface displays the last used Printer as a selection. The User then confirms the selection of the last used Printer.

The last used Printer may be automatically selected by the Client User Interface and may be affected by the current network topology or geo-location, for example the last used Printer may be tracked on a per-network (e.g. default router or other criteria), per-location (e.g. geo-location), or per-Service (e.g. current local server) basis.

Select Printer Using Name or Address

The Client User Interface asks the User for a name or address for the Printer. The User then provides a Printer name or address through the Client User Interface. Finally, the Client User Interface queries the Printer for valid Service URIs.

The Printer name can be a DNS-SD Service name, a fully-qualified domain name, or other unique identifying name. The Printer address can be a numeric IP address or other unique identifying number.

Select Printer Using URI

The Client User Interface asks the User for a Service URI for the Printer. The User then provides a URI through the Client User Interface or cancels selection.

For example, the user could supply an IPP URI: "ipp://example.com/port1" as reported by the Printer's network configuration page.

Select Printer Using a Directory Service

The Client obtains a list of Printers on behalf of the User from the Directory Service and validates that each Printer supports one or more Client-supported Service protocols. The Client User Interface then asks the User to select one of the supported Printers. Finally, the User selects a Printer.

Preconditions: One or more Printers are listed in a Directory Service and that Directory Service is Visible to the Client.

Select Printer Using a Cloud Service

The Client obtains a list of Printers on behalf of the User from the Cloud Service(s). The Client User Interface then asks the User to select one of the Printers. Finally, the User selects a Printer.

Preconditions: The Client and one or more Printers are registered with a Cloud Service, and that Cloud Service is Visible to both the Client and Printers. The Client and Printers may be registered with multiple Cloud Services, and both may maintain multiple identities for a particular Cloud Service.

Select Printer Using a Discovery Protocol

The Client initiates Discovery on behalf of the User and maintains a dynamic list of Visible Printers during selection. The Client User Interface asks the user to select one of the Visible Printers, updating those Printers as they come and go. Finally, the User selects a Printer and the Client terminates Discovery.

Preconditions: The Printer is Visible to the Client and supports a common Discovery Protocol.

Select Printer Using Geo-Location

The Client initiates a proximity detection of nearby Printers using Services and/or Discovery Protocols, hiding duplicate Printers that are reported by multiple Service and/or Discovery Protocols. The Client User Interface asks the User to select one of the nearby Printers. Finally, the User selects a nearby Printer.

Preconditions: Both the Client and Printer have access to geo-location information to allow for proximity detection, and both support common Discovery Protocol(s).

Select Printer Using Out of Band Method

The User asks the Client User Interface to identify the Printer. The Client initiates identification to obtain a Service URI and descriptive information. The Client User Interface then asks the User to confirm the selection of the identified Printer. Finally, the User confirms the selection.

Precondition: The Printer and Client support a common identifying technology such as near-field communications, QRCodes, or bar codes.

Select Printer Using Properties

A User selects a Printer using properties such as Service, capability, or description properties of the Printer. Service properties include the application protocol, security, or restrictions such as the maximum number of pages allowed in a job. Capability properties include values such as media, duplex, finishing, color support, and so forth, Description properties include values such as location, speed, color support, and job size. The properties may be provided by a combination of User input, policy, and/or software heuristic.

The User asks the Client User Interface to select using properties. The Client obtains a list of Printers for the User that meet the given properties provided by the Client software, policy, and/or User and validates that each Printer supports one or more Client-supported Service protocols. The Client User Interface then asks the User to select one of the supported Printers. Finally, the User selects a Printer.

Print

Printing is a common (perhaps the most common) use case for Printers. Each of the use cases in this section begin by initiating a print, selecting a Printer (section 3.2.1), querying the Printer status, capabilities, and status information, and displaying of any status information important to the User. Each use case generally ends with the User collecting the printout from the Printer.

Preconditions: For all of the following use cases, the Printer must be Visible to the Client in order to be selected, either directly or through an intermediate Service. Also, the document to be printed must be Visible to the Printer and in a format suitable for the Printer or converted by the Client (or Service) into a suitable format.

Print a Document

User has a Client connected to the Wi-Fi network in her business and has a document to print prior to a meeting that is stored on her phone.

After the User initiates a print and selects a Printer, the User specifies the processing intent for the Job and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and local document. The Printer validates the Job Ticket and document data and then prints the document.

Print a Document by Reference

User has a Client connected to the Wi-Fi network in her business and is viewing a document on a server that she would like to print.

After the User initiates a print and selects a Printer, the User specifies the processing intent for the Job and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and remote document URI. The Printer validates the Job Ticket and document URI and then prints the document.

Print Using Loaded Media

User is viewing a photo and would like to print the photo on the largest borderless photographic media loaded on her Printer.

After the User initiates a print from the phone and selects a Printer, the Client automatically selects the largest borderless photographic media loaded on the Selected Printer and the highest print quality. The User selects additional processing intent for the Job and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and local photo. The Printer validates the Job Ticket and document data and then prints the photo.

Preconditions: Printer can report loaded media information such as size, type, coating, and weight. This may be detected automatically or manually entered by the User or Operation when loading the media.

Print a Secure Form

The treasurer of a small training company that is holding a meeting and seminar at a resort needs to print out 20 checks for training personnel.  He uses an accounting program to enter the hours worked, bonuses, reimbursable expenses, and so forth and prints the checks on a printer provided by the resort using check blanks he brought to the meeting.

The User loads check blanks into the Printer and configured the loaded media as necessary at the Printer. After the User initiates a print from his accounting program, selects a Printer for printing, and selects checks to be printed, the Client User Interface displays a preview of the printed checks and the User confirms that checks amounts, payees and signature are correct. The Client automatically selects the check blank media. The User selects additional processing intent for the Job and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and document data containing the checks, correctly oriented for the check blank media. The User waits for the checks to be printed and removes any excess media from the Printer.

Print with Special Formatting

At a seminar located at a country resort, a factotum and general gofer has been asked to provide 80 sets of ten keywords/phrases, clearly printed on 2-inch by 1-inch paper slips for use in a get acquainted exercise. Costs are to be minimized. Gofer has a laptop with a word processer program. Resort has a Wi-Fi network available to users and a networked MFD at the business center. Attendant at business center will charge for any printed sheets removed from premises.

After the User initiates a print from his word processor and selects a Printer, the User selects the processing intent for the Job and confirms the print action. The Client produces document data using the media information (size and margins) in the Job Ticket so that 2-inch by 1-inch slips are spread evenly over each page and sends a print job request to the Printer with the Job Ticket and document. The Printer validates the Job Ticket and document data and then prints the document.

Print and Select at Printer

One or more Printers are integrated with a Service to provide "follow me" printing. The User may release a job at a given Printer by providing a PIN and/or other unique identification/authorization information such as a username and password or ID card.

After initiating a print and selecting a Service, the User specifies the processing intent and PIN for the Job and confirms the print action. The Client sends a print job request to the Service with the Job Ticket and local document. The Service validates the Job Ticket and document data and then holds the document until released by the User at the Printer.

Precondition: The Client and Printer support a common authorization or identification system.

Print to a Service

John is flying to New York for a presentation and doesn't want to carry the presentations. John arrives in New York and goes online from his mobile phone. He selects a local print provider after reviewing the provider web pages and submits his document for printing. He specifies that he needs 10 color copies, printed duplex and stapled on the left side. He also specifies the covers to be 80lb. stock, and the internal pages to be 24lb. stock. John arrives at the provider and picks up his presentations, paying with his corporate credit card.

Print to a Recipient

The recipient may release a job at a given Printer by providing a PIN and/or other unique identification/authorization information such as a username and password or ID card.

After initiating a print and selecting a Printer, the User specifies the processing intent, specifies a recipient, and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and local document. The Printer validates the Job Ticket and document data and then holds the document until released by the recipient. Finally, the recipient collects the printout from the Printer.

Print with a Proof Copy

After initiating a print and selecting a Printer, the User specifies the processing intent, requests a proof print, and confirms the print action. The Client sends a print job request to the Printer with the Job Ticket and local document. The Printer validates the Job Ticket and document data and then prints a proof copy of the document. The User collects the proof printout from the Printer and verifies correct output. The User then initiates a full print of the document from the Client or Printer to produce the final printout.

Exceptions

Select Printer Canceled

The User cancels selection of a Printer. This may also cancel the print request.

Printer No Longer Visible after Selection

After selecting a Visible Printer, the Client, Printer, or network suffers a failure preventing the Client from communicating with the Printer. Typically this will display an error message on the Client and cancel the print request.

Not Authorized

After confirming the print request, the Printer responds that the User is not authorized to print the document. The reason for the authorization failure may involve general access to the Printer or disallowed Job Ticket values, for example a User may not be allowed to print in color.

Not Authenticated

After confirming the print request or selecting the Printer, the User is asked to authenticate with the Printer in order to gain access.

Not Accepting Jobs

After confirming the print request, the Client discovers that the Printer is no longer accessing jobs, displays an error message, and cancels the print request.

Job Ticket or Document Format Not Supported

After confirming the print request, the Printer rejects the request because the job ticket or document format is not supported. The Client displays an error message and cancels the print request.

Job or Document Processing Failures

While processing a job, the Printer reports job or document processing issues to the Client, which displays an error message as needed and asks the User or Operator to confirm the disposition of the Job. Processing failures include out-of-memory, missing resource, and other conditions that prevent a particular Job or document from printing.

Printer Fault

While processing a Job, the Printer reports printer faults to the Client, which displays an error message as needed and asks the User or Operator to confirm the disposition of the Job. Printer faults include "out of paper" and other conditions that stop the processing of Jobs.

Printer Warning

While processing a Job, the Printer reports printer warnings to the Client, which displays a warning message as needed. Printer warnings include "low toner" and other advisory conditions that do not stop the processing of Jobs and do not require immediate attention.

Out of Scope

TBD

Design Requirements

The IPP Everywhere design should:

- Define conformance profiles that reference the IPP/2.0 versions [PWG5100.12];

- Follow the naming conventions defined in IETF IPP/1.1 [RFC2911], including keyword value case (lower) and hyphenation requirements;

- Define conformance requirements for both Printers and Clients; and

- Support "driverless" printing from any client to any Printer using a variety of discovery protocols, IPP for the transport, and standard document formats.

Discovery Protocols

IPP Everywhere Printers MUST support DNS-SD-based discovery and MAY support discovery via other protocols such as LDAP, SLP, and SSDP.

Printer Description Attributes Used in Discovery

Table 1 lists the IPP Printer Description attributes that would normally be used for discovery or coloring of discovered printers.

Table 1 - Printer Description Attributes vs. Discovery Protocols

| IPP Attribute | DNS-SD TXT Key | LDAP/SLP Attribute |
| --- | --- | --- |
| color-supported | Color | printer-color-supported |
| copies-supported | Copies | printer-copies-supported |
| document-formats-supported | pdl | printer-document-format-supported |
| finishings-supported | Bind, Punch, Sort, Staple | printer-finishings-supported |
| ipp-versions-supported | (subtype) | printer-ipp-versions-supported |
| media-supported | PaperCustom, PaperMax | printer-media-supported |
| multiple-document-handling | Collate | - |
| pages-per-minute | - | printer-pages-per-minute |
| pages-per-minute-color | - | printer-pages-per-minute-color |
| printer-charge-info-uri | chargeuri | printer-charge-info-uri |
| printer-device-id | usb_CMD, usb_MDL, usb_MFG | printer-device-id (note 1) |
| printer-geo-location | (LOC record) | printer-geo-location (note 1) |
| printer-info | (instance) | printer-info |
| printer-location | note | printer-location |
| printer-make-and-model | ty | printer-make-and-model |
| printer-more-info | adminurl | printer-more-info |
| printer-name | (instance) | printer-name |
| printer-organization | - | printer-organization (note 1) |
| printer-organizational-unit | - | printer-organizational-unit (note 1) |
| printer-uri-supported | (service + host + port) rp | printer-uri, printer-xri-supported |
| printer-uuid | UUID | printer-uuid (note 1) |
| sides-supported | Duplex | printer-sides-supported |
| uri-authentication-supported | air | printer-xri-supported |
| uri-security-supported | TLS | printer-xri-supported |

Note 1: Extension attribute to RFC 3712.

DNS Service Discovery

DNS Service Discovery uses SRV records and traditional unicast and multicast DNS queries. This discovery protocol is collectively defined in the Bonjour Printing Specification version 1.0.2 [BONJOUR] and extended in this specification.

Service Instance Name (SRV)

IPP Everywhere Printers MUST NOT use a service instance name containing a unique identifier by default. A unique identifier may only be added to the instance if there is a name collision.

IPP Everywhere Printers MUST advertise the "_ipp._tcp" (generic IPP) and  "_ipp._tcp,_print" (IPP Everywhere Print) services.

IPP Everywhere Printers supporting the "ipps" URI scheme [IPPS] MUST advertise the "_ipps._tcp" (generic IPPS) and "_ipps._tcp,_print" (IPP Everywhere Secure Print) services.

The domain portion of the service instance name MUST BE "local." for multicast DNS.

Geo-Location (LOC)

Printers MUST publish LOC records [RFC1876] to provide the physical location of the Printer. Printers MUST allow the user to configure the geo-location manually.

Service Information (TXT)

Printers MUST publish a TXT record for each domain that is served. The following subsections define new key/value pairs in addition to those required by the Bonjour Printing Specification [BONJOUR]. Table 2 lists all of the key/value pairs which are REQUIRED when the corresponding key has a non-default value. Printers SHOULD omit key/value pairs when the value matches the default value for the corresponding key.

Table 2 - DNS TXT Record Keys

| Key | Description | Default Value |
| --- | --- | --- |
| adminurl | The Printer-resident configuration page URL as reported by the "printer-more-info" Printer Description attribute. | '' (empty string) |
| air | The type of authentication information that is required for the Printer. See section 4.2.3.1. | 'none' |
| Bind | 'T' if the Printer can bind output, 'F' otherwise. | 'U' (note 1) |
| Collate | 'T' if the Printer can collate copies, 'F' otherwise. | 'U' (note 1) |
| Color | 'T' if the Printer supports color printing, 'F' otherwise. | 'U' (note 1) |
| Copies | 'T' if the Printer can make copies on its own, 'F' otherwise. | 'U' (note 1) |
| Duplex | 'T' if the Printer supports duplex printing, 'F' otherwise | 'U' (note 1) |
| note | The location of the Printer as reported by the "printer-location" Printer Description attribute. | '' (empty string) |
| PaperCustom | 'T' if the Printer supports custom media sizes, 'F' otherwise. | 'U' (note 1) |
| PaperMax | The maximum media size supported by the Printer: '<legal-A4', 'legal-A4', 'isoC-A2', '>isoC-A2'. | 'legal-A4' |
| pdl | A comma-delimited list of supported MIME media types. See section 4.2.3.2. | '' (empty string) |
| priority | The priority for the service from 0 to 99, where 0 is the highest priority and 99 is the lowest priority. | '50' |
| product | The PostScript product name, typically the value reported by the "printer-make-and-model" Printer Description attribute with parenthesis, e.g. '(Example Model)'. | '' (empty string) |
| Punch | 'T' if the Printer can punch output, 'F' otherwise. | 'U' (note 1) |
| qtotal | The number of queues for this Printer which MUST have the value '1'. | '1' |
| rp | The remote print queue name, which is the resource path portion of the Printer URI without the leading slash. | '' (empty string) |
| Sort | 'T' if the Printer can sort output, 'F' otherwise. | 'U' (note 1) |
| Staple | 'T' if the Printer can staple output, 'F' otherwise. | 'U' (note 1) |
| TLS | The maximum TLS version supported or 'none' if no version of TLS is supported. See section 4.2.3.3. | 'none' |
| txtvers | The major version of the Bonjour printing specification. MUST have the value '1'. | '1' |
| ty | The make and model of the printer as reported by the "printer-make-and-model" Printer Description attribute. | '' (empty string) |
| usb_CMD | The IEEE 1284 Device ID command set value. See section 4.2.3.4. | '' (empty string) |
| usb_MDL | The IEEE 1284 Device ID model value. See section 4.2.3.6. | '' (empty string) |
| usb_MFG | The IEEE 1284 Device ID manufacturer value. See section 4.2.3.5. | '' (empty string) |
| UUID | The UUID of the Printer without the 'urn:uuid:' prefix as reported by the "printer-uuid" Printer Description attribute. See section 4.2.3.7. | '' (empty string) |

Note 1: The value 'U' means "undefined".

air

The "air" key defines the type of authentication information that is required for imaging. The name "air" comes from the CUPS "auth-info-required" Printer Description attribute [CUPSIPP] which extends the "uri-authentication-supported" Printer Description attribute [RFC2911]. The following values are supported:

'certificate'; Authentication using SSL/TLS certificates. This is equivalent to the value 'certificate' for the "uri-authentication-supported" Printer Description attribute [RFC2911].

'negotiate'; Kerberized authentication is required [RFC4559]. This is equivalent to the 'negotiate' value for the "uri-authentication-supported" Printer Description attribute [PWG5100.JPS3].

'none'; No authentication is required. This is equivalent to the value 'none' for the "uri-authentication-supported" Printer Description attribute [RFC2911].

'username,password'; Username + password authentication is required. This is equivalent to the values 'basic' or 'digest' for the "uri-authentication-supported" Printer Description attribute [RFC2911].'negotiate'; Kerberized authentication is required [RFC4559] [PWG5100.JPS3]. There is no equivalent value for the "uri-authentication-supported" Printer Description attribute.

The default value for the "air" key is 'none'.

pdl

The REQUIRED "pdl" key lists the supported MIME media types. Because the total length of a key/value pair is 255 octets, the "pdl" value is typically a subset of the values reported by the "document-format-supported" Job Template attribute. Printers SHOULD populate the "pdl" key with a comma-delimited list of the preferred MIME media types and MUST NOT list the 'application/octet-stream' MIME media type.

TLS

The "TLS" key defines the highest version of TLS that is supported for encrypted communications with the Printer. The following values are supported:

'none'; No encryption is supported. This is equivalent to the value 'none' for the "uri-security-supported" Printer Description attribute.

'1.0'; TLS 1.0 [RFC2246] encryption is supported. This is equivalent to the value 'tls' for the "uri-security-supported" Printer Description attribute.

'1.1'; TLS 1.1 [RFC4346] encryption is supported. This is equivalent to the value 'tls' for the "uri-security-supported" Printer Description attribute.

'1.2'; TLS 1.2 [RFC5246] encryption is supported. This is equivalent to the value 'tls' for the "uri-security-supported" Printer Description attribute.

The default value of the "TLS" key is 'none'.

usb_CMD

The REQUIRED "usb_CMD" key provides the command set [PWG5107.2] value from the "printer-device-id" Printer attribute.

usb_MFG

The REQUIRED "usb_MFG" key provides the manufacturer value from the "printer-device-id" Printer attribute.

usb_MDL

The REQUIRED "usb_MDL" key provides the model name value from the "printer-device-id" Printer attribute.

UUID

The REQUIRED "UUID" key provides the value of the "printer-uuid" Printer Description attribute [RFC4122] [PWG 5100.JPS3] without the leading "urn:uuid:". For example, if a printer reports a "printer-uuid" value of:

urn:uuid:12345678-9ABC-DEF0-1234-56789ABCDEF0

The "UUID" key will have a value of:

12345678-9ABC-DEF0-1234-56789ABCDEF0

LDAP and SLP Discovery

LDAP and SLP discovery use the schema defined in Lightweight Directory Access Protocol (LDAP): Schema for Printer Services [RFC3712] [RFC4511] [RFC4515].

SSDP Discovery

The Simple Service Discovery Protocol (SSDP) is a multicast UDP protocol used to support discovery of Universal Plug-and-Play [UPNP1.1] network devices and services. This document defines a new device template named "Printer:2" that is compliant with the UPnP Device Architecture, Version 1.1. The goal of Printer:2 is to modify the Printer Device as minimally as possible to support the discovery of IPP Everywhere Printers. In this case UPnP is being used as Discovery Service for an IPP Everywhere Printer and provides basic information about the capabilities of a Print class device without having to query the Printer's IPP attributes. After Discovery, all additional communication with the device is done via IPP.

Figure 1 - UPnP Printer:2 Functional Diagram

Figure 1 illustrates a Printer device with the required service being the IPP Everywhere Service. The Printer Device includes the location of the IPP Everywhere endpoints for communicating with the device. The Printer:2 advertisement includes the equivalent of the DNS-SD TXT records for the Printer discovered with Printer:2.

The IPP Everywhere Service is REQUIRED in a Printer:2 Device. Additional services MAY be present such as the optional PrintBasic service. The PrintBasic Service includes print-related attributes along with all Job-related attributes (e.g., JobName, DocumentFormat, Copies, etc). The optional PrintEnhancedLayout Service extends the PrintBasic service with additional operations and semantics. Both PrintBasic and PrintEnhancedLayout are OPTIONAL and provided only for legacy device support.

Device Definitions

Device Type

The following device type identifies a device that is compliant with this template:

urn:schemas-upnp-org:device:printer:2

Device Model

Products that expose devices of the type printer:2 must implement minimum version numbers of all required embedded devices and services specified in Table 3.

Table 3 - Device Requirements for urn:schemas-upnp-org:printer:2

| Device Type | Root | Req or Opt (note 1) | Service Type | Req or Opt (note 1) | Service ID (note 2) |
| --- | --- | --- | --- | --- | --- |
| Printer:2 | Root | R | PrintBasic:1 | O | 1 |
|  |  |  | PrintEnhancedLayout:1 | O | 2 |
|  |  |  | IPPEverywhere:1 | R | 3 |

Note 1: R = Required, O = Optional, X = Non-standard.

Note 2: Prefixed by urn:upnp-org:serviceId:

Description of Device Requirements

The IPPEverywhere Service is a required service for a Printer:2 Device.

Relationships Between Services

The Printer Device only REQUIRES one service, the IPPEverywhere Service. When PrintBasic or PrintEnhancedLayout are implemented, they are independent of IPPEverywhere, although PrintEnhancedLayout has a strong tie to PrintBasic. Again, these optional services are only present for backward compatibility

Theory of Operation

Discovery

When a device is added to the network, the UPnP discovery protocol allows that device to advertise its services to user control points on the network. The UPnP discovery protocol is based on SSDP. The Printer device must announce itself as:

- a root device

a device UUID for its device type

a device type

Job Submission and Control

Job Submission and Control are handled through standard IPP operations.

XML Device Description

The following XML describes a typical IPP Printer device:

<?xml version="1.0"?>

<root xmlns="urn:schemas-upnp-org:device-1-0">

<specVersion>

<major>1</major>

<minor>0</minor>

</specVersion>

<URLBase>base URL for all relative URLs</URLBase>

<device>

<deviceType>urn:schemas-upnp-org:device:Printer:2</deviceType>

<friendlyName>printer-info string</friendlyName>

<manufacturer>manufacturer name</manufacturer>

<manufacturerURL>URL to manufacturer site</manufacturerURL>

<modelDescription>printer-make-and-model string</modelDescription>

<modelName>model name</modelName>

<modelNumber>model number</modelNumber>

<modelURL>URL to model site</modelURL>

<serialNumber>manufacturer's serial number</serialNumber>

<UDN>urn:uuid:UUID</UDN>

<UPC>Universal Product Code</UPC>

<iconList>

<icon>

<mimetype>image/format</mimetype>

<width>horizontal pixels</width>

<height>vertical pixels</height>

<depth>color depth</depth>

<url>URL to icon</url>

</icon>

XML to declare other icons, if any, go here.

</iconList>

<serviceList>

<service>

<serviceType>urn:schemas-pwg.org:service:IPPEverywhere:1

</serviceType>

<serviceId>urn:upnp-org:serviceId:3</serviceId>

<adminurl>printer-more-info string</adminurl>

<note>printer-location string</note>

<air>Type of authentication required for the services.</air>

<tls>Version of TLS supported.</tls>

<rp>This key is used to specify the print queue name.</rp>

<pdlp>Comma-separated list of MIME types supported by printing.</pdlp>

<Bind>Set value to “T” if the printer is capable of binding its output, “F” if its not.</Bind>

<Collate>Set value to “T” if the printer is capable of generating collated copies, “F” if its not.</Collate>

<Color>Set value to “T” if the printer is capable of generating color output, “F” if its not.</Color>

<Copies>Set value to “T” if the printer is capable of generating fast copies, “F” if its not.</Copies>

<Duplex>Set value to “T” if the printer is capable of generating two-sided output, “F” if its not.</Duplex>

<PaperCustom>Set value to “T” if the printer can handle custom paper sizes, “F” if its not.</PaperCustom>

<PaperMax> “less-than-legal-A4”, “legal-A4”, “isoC-A2”, or “greater-than-isoC-A2”</PaperMax>

<Punch> Set value to the number of holes of the hole puncher supported by the printer.</Punch>

<Sort> Set value to “T” if the printer is capable of sort- ing its output, “F” if its not.</Sort>

<Staple> Set value to “T” if the printer is capable of sta- pling output, “F” if its not.</Sort>

</service>

</serviceList>

<deviceList>

Description of embedded devices added by UPnP vendor (if any) go here.

</deviceList>

<presentationURL>URL for presentation</presentationURL>

</device>

</root>

Protocol Binding

Printers and Clients MUST support IPP/2.0, IPP/2.1, and/or IPP/2.2 as defined in Internet Printing Protocol 2.0 Second Edition [PWG5100.12] and the IPP Job and Printer Extensions - Set 3 [PWG5100.JPS3].

While this specification defines an IPP binding, the same set of Semantic Elements can be applied to any protocol that conforms to the PWG Semantic Model.

IPP Operations

Table 4 lists the REQUIRED operations for an IPP Everywhere Printer.

Table 4 - Required Operations

| Code | Operation Name | Reference |
| --- | --- | --- |
| 0x0002 | Print-Job | RFC 2911 |
| 0x0004 | Validate-Job | RFC 2911 |
| 0x0005 | Create-Job (note 1) | RFC 2911 |
| 0x0006 | Send-Document (note 1) | RFC 2911 |
| 0x0008 | Cancel-Job | RFC 2911 |
| 0x0009 | Get-Job-Attributes | RFC 2911 |
| 0x000A | Get-Jobs | RFC 2911 |
| 0x000B | Get-Printer-Attributes | RFC 2911 |
| 0x0039 | Cancel-My-Jobs (note 2) | PWG 5100.11 |
| 0x003B | Close-Job (note 2) | PWG 5100.11 |
| 0x003C | Identify-Printer (note 3) | PWG 5100.JPS3 |

Note 1: Operation is newly required for IPP/2.0 Printers.

Note 2: Operation is newly required for IPP/2.0 and IPP/2.1 Printers.

Note 3: Operation is newly required.

IPP Printer Description Attributes

Table 5 lists the REQUIRED Printer Description attributes for an IPP Everywhere Printer.

Table 5 - Printer Description Attributes

| Attribute | Reference |
| --- | --- |
| charset-configured | RFC 2911 |
| charset-supported | RFC 2911 |
| color-supported | RFC 2911 |
| compression-supported | RFC 2911 |
| document-format-default | RFC 2911 |
| document-format-supported | RFC 2911 |
| generated-natural-language-supported | RFC 2911 |
| identify-actions-default (note 3) | PWG 5100.JPS3 |
| identify-actions-supported (note 3) | PWG 5100.JPS3 |
| ipp-extensions-supported (note 3) | PWG 5100.JPS3 |
| ipp-versions-supported | RFC 2911 |
| job-constraints-supported (note 3) | PWG 5100.JPS3 |
| job-creation-attributes-supported (note 2) | PWG 5100.11 |
| job-ids-supported (note 2) | PWG 5100.11 |
| job-password-encryption-supported (note 5) | PWG 5100.11 |
| job-password-supported (note 5) | PWG 5100.11 |
| job-preferred-attributes-supported (note 3) | PWG 5100.JPS3 |
| job-resolvers-supported (note 3) | PWG 5100.JPS3 |
| media-bottom-margin-supported (note 3) | PWG 5100.JPS3 |
| media-col-database (note 2) | PWG 5100.11 |
| media-col-ready (note 2) | PWG 5100.3 |
| media-col-supported (note 1) | PWG 5100.3 |
| media-left-margin-supported (note 3) | PWG 5100.JPS3 |
| media-ready (note 2) | RFC 2911 |
| media-right-margin-supported (note 3) | PWG 5100.JPS3 |
| media-source-supported (note 3) | PWG 5100.JPS3 |
| media-top-margin-supported (note 3) | PWG 5100.JPS3 |
| media-type-supported (note 3) | PWG 5100.JPS3 |
| multiple-document-jobs-supported (note 1) | RFC 2911 |
| multiple-operation-timeout (note 1) | RFC 2911 |
| multiple-operation-timeout-action (note 2) | PWG 5100.JPS3 |
| natural-language-configured | RFC 2911 |
| operations-supported | RFC 2911 |
| overrides-supported (note 2) | PWG 5100.6 |
| pages-per-minute | RFC 2911 |
| pages-per-minute-color | RFC 2911 |
| printer-alert (note 2) | PWG 5100.9 |
| printer-alert-description (note 2) | PWG 5100.9 |
| printer-charge-info (note 4) | PWG 5100.JPS3 |
| printer-charge-info-uri (note 4) | PWG 5100.JPS3 |
| printer-config-change-date-time (note 3) | PWG 5100.JPS3 |
| printer-config-change-time (note 3) | PWG 5100.JPS3 |
| printer-device-id (note 2) | PWG 5107.2 |
| printer-geo-location (note 3) | PWG 5100.JPS3 |
| printer-get-attributes-supported (note 3) | PWG 5100.JPS3 |
| printer-icc-profiles (note 3) | PWG 5100.JPS3 |
| printer-icons (note 3) | PWG 5100.JPS3 |
| printer-info | RFC 2911 |
| printer-is-accepting-jobs | RFC 2911 |
| printer-location | RFC 2911 |
| printer-make-and-model | RFC 2911 |
| printer-mandatory-job-attributes (note 4) | PWG 5100.JPS3 |
| printer-more-info | RFC 2911 |
| printer-name | RFC 2911 |
| printer-organization (note 3) | PWG 5100.JPS3 |
| printer-organizational-unit (note 3) | PWG 5100.JPS3 |
| printer-state | RFC 2911 |
| printer-state-change-data-time (note 2) | RFC 3995 |
| printer-state-change-time (note 1) | RFC 3995 |
| printer-state-message | RFC 2911 |
| printer-state-reasons | RFC 2911 |
| printer-supply (note 3) | PWG 5100.JPS3 |
| printer-supply-description (note 3) | PWG 5100.JPS3 |
| printer-supply-info-uri (note 3) | PWG 5100.JPS3 |
| printer-up-time | RFC 2911 |
| printer-uri-supported | RFC 2911 |
| printer-uuid (note 3) | PWG 5100.JPS3 |
| pwg-raster-document-resolution-supported (note 3) | PWG 5102.RAS |
| pwg-raster-document-sheet-back (note 3) | PWG 5102.RAS |
| pwg-raster-document-type-supported (note 3) | PWG 5102.RAS |
| queued-job-count | RFC 2911 |
| uri-security-supported | RFC 2911 |
| uri-authentication-supported | RFC 2911 |
| which-jobs-supported (note 2) | PWG 5100.11 |

Note 1: Attribute is newly required for IPP/2.0 Printers.

Note 2: Attribute is newly required for IPP/2.0 and IPP/2.1 Printers.

Note 3: Attribute is newly required.

Note 4: REQUIRED for Printers that implement paid imaging services.

Note 5: REQUIRED for printers that support the "application/openxps" or "application/pdf" MIME media types.

IPP Operation Attributes

Table 6 lists the REQUIRED operation attributes for an IPP Everywhere Printer.

Table 6 - Operation Attributes

| Attribute | Reference |
| --- | --- |
| compression | RFC 2911 |
| document-format | RFC 2911 |
| document-format-version (note 1) | PWG 5100.7 |
| first-index (note 3) | PWG 5100.JPS3 |
| first-job-id | RFC 2911 |
| identify-actions (note 3) | PWG 5100.JPS3 |
| job-ids (note 2) | PWG 5100.11 |
| last-document (note 1) | RFC 2911 |
| limit | RFC 2911 |
| requesting-user-name | RFC 2911 |
| which-jobs (note 2) | RFC 2911, PWG 5100.11 |

Note 1: Attribute is newly required for IPP/2.0 Printers.

Note 2: Attribute is newly required for IPP/2.0 and IPP/2.1 Printers.

Note 3: Attribute is newly required.

IPP Job Template Attributes

Table 7 lists the REQUIRED Job Template attributes for an IPP Everywhere Printer.

Table 7 - Job Template Attributes

| Attribute | Reference |
| --- | --- |
| copies (note 5) | RFC 2911 |
| copies-default (note 5) | RFC 2911 |
| copies-supported (note 5) | RFC 2911 |
| finishings (note 7) | RFC 2911 |
| finishings-default (note 7) | RFC 2911 |
| finishings-supported (note 7) | RFC 2911 |
| ipp-attribute-fidelity | RFC 2911 |
| job-accounting-user-id (note 4) | PWG 5100.3 |
| job-billing-info (note 4) | PWG 5100.3 |
| job-mandatory-attributes (note 2) | PWG 5100.11 |
| job-name | RFC 2911 |
| job-password (note 6) | PWG 5100.11 |
| job-password-encryption (note 6) | PWG 5100.11 |
| media | RFC 2911 |
| media-col (note 1) | PWG 5100.3 |
| media-col.media-bottom-margin (note 3) | PWG 5100.JPS3 |
| media-col.media-left-margin (note 3) | PWG 5100.JPS3 |
| media-col.media-right-margin (note 3) | PWG 5100.JPS3 |
| media-col.media-size (note 1) | PWG 5100.3 |
| media-col.media-source (note 3) | PWG 5100.JPS3 |
| media-col.media-top-margin (note 3) | PWG 5100.JPS3 |
| media-col.media-type (note 1) | PWG 5100.3 |
| media-col-default (note 1) | PWG 5100.3 |
| media-default | RFC 2911 |
| media-supported | RFC 2911 |
| multiple-document-handling (note 6) | RFC 2911 |
| orientation-requested | RFC 2911 |
| orientation-requested-default | RFC 2911 |
| orientation-requested-supported | RFC 2911 |
| output-bin | PWG 5100.2 |
| output-bin-default | PWG 5100.2 |
| output-bin-supported | PWG 5100.2 |
| overrides (note 2) | PWG 5100.6 |
| page-ranges (note 6) | RFC 2911 |
| page-ranges-supported (note 6) | RFC 2911 |
| print-color-mode (note 3) | PWG 5100.JPS3 |
| print-color-mode-default (note 3) | PWG 5100.JPS3 |
| print-color-mode-supported (note 3) | PWG 5100.JPS3 |
| print-content-optimize (note 3) | PWG 5100.7 |
| print-content-optimize-default (note 3) | PWG 5100.7 |
| print-content-optimize-supported (note 3) | PWG 5100.7 |
| print-rendering-intent (note 3) | PWG 5100.JPS3 |
| print-rendering-intent-default (note 3) | PWG 5100.JPS3 |
| print-rendering-intent-supported (note 3) | PWG 5100.JPS3 |
| print-quality | RFC 2911 |
| print-quality-default | RFC 2911 |
| print-quality-supported | RFC 2911 |
| printer-resolution | RFC 2911 |
| printer-resolution-default | RFC 2911 |
| printer-resolution-supported | RFC 2911 |
| sides | RFC 2911 |
| sides-default | RFC 2911 |
| sides-supported | RFC 2911 |

Note 1: Attribute is newly required for IPP/2.0 Printers.

Note 2: Attribute is newly required for IPP/2.0 and IPP/2.1 Printers.

Note 3: Attribute is newly required.

Note 4: REQUIRED for Printers that implement paid imaging services.

Note 5: REQUIRED for printers that support the "application/openxps", "application/pdf", or "image/jpeg" MIME media types.

Note 6: REQUIRED for printers that support the "application/openxps" or "application/pdf" MIME media types.

Note 7: REQUIRED for printers with finishers.

IPP Job Description Attributes

Table 8 lists the REQUIRED Job Description attributes for an IPP Everywhere Printer.

Table 8 - Job Description Attributes

| Attribute | Source |
| --- | --- |
| compression-supplied (note 1) | PWG 5100.7 |
| copies-preferred (note 3) | PWG 5100.JPS3 |
| date-time-at-completed (note 2) | RFC 2911 |
| date-time-at-creation (note 2) | RFC 2911 |
| date-time-at-processing (note 2) | RFC 2911 |
| document-format-supplied (note 1) | PWG 5100.7 |
| document-format-version-supplied (note 1) | PWG 5100.7 |
| document-name-supplied (note 1) | PWG 5100.7 |
| feed-orientation-preferred (note 3) | PWG 5100.JPS3 |
| finishings-preferred (note 3) | PWG 5100.JPS3 |
| job-id | RFC 2911 |
| job-impressions (note 2) | RFC 2911 |
| job-impressions-completed (note 2) | RFC 2911 |
| job-name | RFC 2911 |
| job-originating-user-name | RFC 2911 |
| job-printer-up-time | RFC 2911 |
| job-printer-uri | RFC 2911 |
| job-state | RFC 2911 |
| job-state-message (note 3) | RFC 2911 |
| job-state-reasons | RFC 2911 |
| job-uri | RFC 2911 |
| job-uuid (note 3) | PWG 5100.JPS3 |
| media-col-preferred (note 3) | PWG 5100.JPS3 |
| media-preferred (note 3) | PWG 5100.JPS3 |
| orientation-requested-preferred (note 3) | PWG 5100.JPS3 |
| output-bin-preferred (note 3) | PWG 5100.JPS3 |
| page-ranges-preferred (note 4) | PWG 5100.JPS3 |
| print-color-mode-preferred (note 3) | PWG 5100.JPS3 |
| print-content-optimize-preferred (note 3) | PWG 5100.JPS3 |
| print-quality-preferred (note 3) | PWG 5100.JPS3 |
| print-rendering-intent-preferred (note 3) | PWG 5100.JPS3 |
| printer-resolution-preferred (note 3) | PWG 5100.JPS3 |
| sides-preferred (note 3) | PWG 5100.JPS3 |
| time-at-completed | RFC 2911 |
| time-at-creation | RFC 2911 |
| time-at-processing | RFC 2911 |

Note 1: Attribute is newly required for IPP/2.0 Printers.

Note 2: Attribute is newly required for IPP/2.0 and IPP/2.1 Printers.

Note 3: Attribute is newly required.

Note 4: REQUIRED for printers that support the "application/openxps" or "application/pdf" MIME media types.

Document Formats

IPP Everywhere Printers MUST support documents conforming to the PWG Raster Format [PWG5102.RAS] ("image/pwg-raster") and JPEG File Information Format Version 1.02 [JFIF] ("image/jpeg").

IPP/2.1 and IPP/2.2 Printers MUST and IPP/2.0 Printers SHOULD support documents conforming to Document management — Portable document format — Part 1: PDF 1.7 [ISO32000] ("application/pdf").

IPP Everywhere Printers SHOULD support documents conforming to the Open XML Paper Specification [ECMA388] ("application/openxps").

Additional Values for Existing Attributes

ipp-extensions-supported (1setOf type2 keyword)

This specification defines the REQUIRED keyword 'ipp-everywhere-1.0' for the "ipp-extensions-supported" Printer attribute.

Conformance Requirements

This section summarizes the Conformance Requirements detailed in the definitions in this document for Clients and Printers.

Conformance Requirements for this Specification

In order for a client and a Printer to claim conformance to this specification a client MUST be able to supply and a Printer MUST support the following:

- DNS Service Discovery as defined in section 4.2

- IPP/2.0 as defined in section 5

- The REQUIRED operations listed in Table 4

- The REQUIRED Printer Description attributes listed in Table 5

- The REQUIRED operation attributes listed in Table 6

- The REQUIRED Job Template attributes listed in Table 7

- The REQUIRED Job Description attributes listed in Table 8

- The REQUIRED document formats listed in section 6

- The 'ipp-everywhere-1.0' value for the "ipp-extensions-supported" Printer Description attribute as defined in section 7.1

Conditional Conformance Requirements for Printer Objects

Printers that support the "image/jpeg" [JFIF] MIME media type MUST support the "copies", "copies-default", and "copies-supported" Job Template attributes as defined in section 5.4.

Printers that support the "application/openxps" [ECMA388] or "application/pdf" [ISO32000] MIME media types MUST support the "job-password-supported" and "job-password-encryption-supported" Printer Description attributes as defined in section 5.2 and the "copies", "copies-default", "copies-supported", "job-password", "job-password-encryption", "multiple-document-handling", "overrides", "page-ranges", and "page-ranges-supported" Job Template attributes as defined in section 5.4.

Printers that provide paid print services MUST support the "printer-mandatory-job-attrbutes" Printer Description attribute as defined in section 5.2 and "job-accounting-user-id" and "job-billing-info" Job Template attributes as defined in section 5.4.

Internationalization Considerations

For interoperability and basic support for multiple languages, conforming implementations MUST support the UTF-8 [RFC3629] encoding of Unicode [UNICODE] [ISO10646].

Security Considerations

The IPP extensions defined in this document require the same security considerations as defined in the IPP Model and Semantics [RFC2911].

IANA Considerations

[Editor's note - replace PWG5100.EVE and PWG5100.JPS3 references below with final document numbers]

Attribute Value Registrations

The keyword attribute values defined in this document will be published by IANA according to the procedures in the IPP Model and Semantics [RFC2911] section 6.1 in the following file:

http://www.iana.org/assignments/ipp-registrations

The registry entries will contain the following information:

Attributes (attribute syntax)

Keyword Attribute Value	Reference

-----------------------	---------

ipp-extensions-supported (1setOf type2 keyword)	[PWG5100.JPS3]

ipp-everywhere-1.0	[PWG5100.EVE]

Additional Values for Existing Attributes

ipp-extensions-supported (1setOf type2 keyword)

This specification defines the REQUIRED keyword 'ipp-everywhere-1.0' for the "ipp-extensions-supported" Printer attribute.

References

Normative References

[ECMA388]	"Open XML Paper Specification", June 2009, Standard ECMA-388, http://www.ecma-international.org/publications/standards/Ecma-388.htm

[ISO32000]	"Document management — Portable document format — Part 1: PDF 1.7", ISO 32000-2008

[JFIF]	E. Hamilton, "JPEG File Interchange Format Version 1.02", September 1992, http://www.w3.org/Graphics/JPEG/jfif3.pdf

[PWG5100.3]	K. Ocke, T. Hastings, "Internet Printing Protocol (IPP): Production Printing Attributes – Set1", PWG 5100.3-2001, February 2001, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippprodprint10-20010212-5100.3.pdf

[PWG5100.7]	T. Hastings, P. Zehler, "Standard for The Internet Printing Protocol (IPP): Job Extensions", PWG 5100.7-2003, October 2003, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippjobext10-20031031-5100.7.pdf

[PWG5100.9]	I. McDonald, C. Whittle, "Internet Printing Protocol (IPP)/ Printer State Extensions v1.0", PWG 5100.9-2009, July 2009, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippstate10-20090731-5100.9.pdf

[PWG5100.11]	T. Hastings, D. Fullman, "IPP: Job and Printer Operations - Set 2", PWG 5100.11-2010, October 2010, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippjobprinterext10-20101030-5100.11.pdf

[PWG5100.12]	R. Bergman, H. Lewis, I. McDonald, M. Sweet, "IPP/2.0 Second Edition", PWG 5100.12-2011, February 2011, ftp://www.pwg.org/pub/pwg/candidates/cs-ipp20-2011MMDD-5100.12.pdf

[PWG5100.JPS3]	M. Sweet, I. McDonald, "IPP: Job and Printer Extensions - Set 3 (JPS3)", PWG 5100.JPS3-YYYY, Month Year, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippjobprinterext3v10-yyyymmdd-5100.JPS3.pdf

[PWG5101.2]	R. Bergman, T. Hastings, "Standard for Media Standardized Names", PWG 5101.2-2002, February 2002, ftp://ftp.pwg.org/pub/pwg/candidates/cs-pwgmsn10-20020226-5101.1.pdf

[PWG5102.RAS]	M. Sweet, "PWG Raster Format", PWG 5102.RAS-YYYY, Month Year, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippraster10-yyyymmdd-5102.RAS.pdf

[PWG5107.1]	R. Bergman, M. Fenelon, I. McDonald, I. Pavicevic, "Printer Port Monitor MIB 1.0", PWG 5107.1-2005, October 2005, ftp://ftp.pwg.org/pub/pwg/candidates/cs-pmpportmib10-20051025-5107.1.pdf

[PWG5107.2]	I. McDonald, "PWG Command Set Format for IEEE 1284 Printer ID v1.0", PWG 5107.2-2010, May 2010, ftp://ftp.pwg.org/pub/pwg/candidates/cs-pmp1284cmdset10-20100531-5107.2.pdf

[RFC2083]	T. Boutell, "PNG (Portable Network Graphics) Specification Version 1.0", RFC 2083, March 1997, http://www.ietf.org/rfc/rfc2083.txt

[RFC2782]	A. Gulbrandsen, P. Vixie, L. Esibov, "A DNS RR for specifying the location of services (DNS SRV)", RFC 2782, February 2000, http://www.ietf.org/rfc/rfc2782.txt

[RFC2911]	T. Hastings, R. Herriot, R. deBry, S. Isaacson, P. Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911, September 2000, http://www.ietf.org/rfc/rfc2911.txt

[RFC3382]	R. deBry, R. Herriot, T. Hastings, K. Ocke, P. Zehler, "Internet Printing Protocol (IPP): The 'collection' attribute syntax", RFC 3382, September 2002, http://www.ietf.org/rfc/rfc3382.txt

[RFC3712]	P. Fleming, I. McDonald, "Lightweight Directory Access Protocol (LDAP): Schema for Printer Services", RFC 3712, February 2004, http://www.ietf.org/rfc/rfc3712.txt

[RFC3805]	R. Bergman, H. Lewis, I. McDonald, "Printer MIB v2", RFC 3805, June 2004, http://www.ietf.org/rfc/rfc3805.txt

[RFC3927]	S. Cheshire, B. Aboba, E. Guttman, "Dynamic Configuration of IPv4 Link-Local Addresses", RFC 3927, May 2005, http://www.ietf.org/rfc/rfc3927.txt

[RFC3995]	R. Herriot, T. Hastings, "IPP Event Notifications and Subscriptions", RFC 3995, March 2005, http://www.ietf.org/rfc/rfc3955.txt

[RFC4122]	P. Leach, M. Mealling, R. Salz, "A Universally Unique IDentifier (UUID) URN Namespace", RFC 4122, July 2005, http://www.ietf.org/rfc/rfc4122.txt

[RFC4519]	A. Sciberras, "Lightweight Directory Access Protocol (LDAP): Schema for User Applications", RFC 4519, June 2006, http://www.ietf.org/rfc/rfc4519.txt

[RFC5870]	A. Mayrhofer, C. Spanring, "A Uniform Resource Identifier for Geographic Locations ('geo' URI)", RFC 5870, June 2010, http://www.ietf.org/rfc/rfc5870.txt

[UPNP1.1]	"UPnP™ Device Architecture 1.1", October 2008, http://www.upnp.org/specs/arch/UPnP-arch-DeviceArchitecture-v1.1.pdf

[WGS84]	National Geospatial-Intelligence Agency, "Department of Defense World Geodetic System 1984, Its Definition and Relationships With Local Geodetic Systems, Third Edition", NIMA Technical Report TR8350.2, January 2000,
http://earth-info.nga.mil/GandG/publications/tr8350.2/wgs84fin.pdf

[X.520]	International Telecommunication Union, "Information technology - Open Systems Interconnection - The Directory: Selected attribute types", ITU-T X.520, November 2008.

Informative References

[LDAPSCHEMA]	I. McDonald, "Lightweight Directory Access Protocol (LDAP): Schema for Printer Services",

[MDNS]	S. Cheshire, M. Krocmal, "Multicast DNS", Internet Draft, http://files.multicastdns.org/draft-cheshire-dnsext-multicastdns.txt

Authors' Addresses

Michael Sweet

Apple Inc.

10431 N. De Anza Blvd.

MS 38-4LPT

Cupertino CA 95014

Ira McDonald

High North

PO Box 221

Grand Marais, MI 49839

Andrew Mitchell

Hewlett Packard Company

Send comments to the PWG IPP Mailing List:

ipp@pwg.org (subscribers only)

To subscribe, see the PWG web page:

http://www.pwg.org/

Implementers of this specification document are encourages to join the IPP Mailing List in order to participate in any discussions of clarification issues and review of registration proposals for additional attributes and values.

The editor would like to especially thank the following individuals who also contributed significantly to the development of this document:

Jerry Thrasher - Lexmark

Peter Zehler - Xerox

Change History

[ PWG Secretary: This section must be removed when Document is approved ]

September 27, 2011

- Updated title to "IPP Everywhere 1.0"

Updated terminology

Updated references for OpenXPS and Bonjour

Filled in use cases from common use case document

Updated DNS SD (Bonjour) TXT record information.

Added SSDP/UPNP information from Andrew Mitchell's May 2011 whitepaper

Revamped transport binding section and added new content from JPS3 and PWG Raster

Added conformance requirements

Added security considerations

Added IANA considerations

August 3, 2011

- Updated document status (interim)

- Added OpenXPS and Port Monitor MIB references

- Added usb_* key definitions

- Added placeholder for UPNP

- Cleaned up discovery attribute table

- Added ipp-extensions-supported keyword definition.

- Other changes per April 2011 F2F review (see posted minutes)

March 16, 2011

Initial revision.