## Example attribute specifications:

    - SYSTEM HEALTH ID TLV

This attribute provides the identifier of the System Health Agent (SHA) or the System Health Validator (SHV) generating the SOH or SOHR report entry. For additional information, refer to [MS-SOH] clause 2.2.3.1.

| 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | R | TLV Type |  |  |  |  |  |  |  |  |  |  |  |  |  | Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IANA SMI Vendor Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Component ID |  |  |  |  |  |  |  |

M: The M bit must be set to 0.

R: The R bit is reserved, and must be set to 0

TLV Type: The TLV Type for this packet type must be set to 2.

Length: The Length field for this packet type must be set to 4.

IANA SMI Vendor Code: The Vendor Code must be set to the PWG SMI, which is 2699 (0xA8B).

Component ID: This 8 bit field identifies the component type, qualified by the Vendor Code.  The following values are defined for hard copy devices.  Values are additive (for example, printer + scanner + copier = 7).

1 = Printer Device
2 = Scanner Device
4 = Copier Device
8 = Fax Device

Values 16, 32, 64, and 128 are reserved and must be set to 0. Therefore, valid values for Component ID range from 0x01 to 0x0F.

    - HCD CONFIGURATION FLAGS SUB-TLV

This attribute provides the current state of the Boolean configuration parameters.  For additional information, refer to [HCD-ATR] clause 4.5.1.

| 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | R | TLV Type |  |  |  |  |  |  |  |  |  |  |  |  |  | Length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Configuration Flags |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

M: The M bit must be set to 0.

R: The R bit is reserved, and must be set to 0

TLV Type: The TLV Type for this packet type must be set to 2.

Length: The Length field for this packet type must be set to 4.

Configuration Flags: This 32 bit field represents configuration parameters of the HCD using pairs of bits. One bit of each pair indicates whether or not the parameter is supported, and the second bit of each pair indicates the state of that configuration parameter:

| Bit | Parameter | Value = 0 | Value = 1 |
| --- | --- | --- | --- |
| 00 | Forwarding | Not enabled | Enabled |
| 01 |  | Not supported | Supported |
| 02 | Administrative Password | Not configured | Configured |
| 03 |  | Not supported | Supported |
| 04 | PSTN Fax | Not enabled | Enabled |
| 05 |  | Not supported | Supported |
| 06 | Authenticated time | Not enabled | Enabled |
| 07 |  | Not supported | Supported |

If a parameter is not supported, then its state SHOULD be set to 0 for transmission and ignored upon receipt. Bits 08 through 31 are Reserved and MUST be set to 0 for transmission and ignored upon receipt.